### Make Your Own Penetrating Oil

by admin

In case of a major change in our life style that resulted in a collapse of society, we would be forced to improvise many things that we now take for granted. Just simple things like loosening a rusty bolt would be a challenge to many of us, not being experienced mechanics.

Yesterday a friend sent me information on a study performed by Machinist’s Workshop magazine in their April 2007 issue. They looked at different penetrating oils to see which one did the best job of removing a rusted bolt by measuring the pounds of torque required to loosen the bolt once treated.

They arranged a subjective test of all the popular penetrants with the control being the torque required to remove the nut from a “scientifically rusted” environment.

Penetrating oils ……….. Average torque load to loosen

- No Oil used ………………. 516 pounds

- WD-40 ………………… … 238 pounds

- PB Blaster ……………….. 214 pounds

- Liquid Wrench ……………127 pounds

- Kano Kroil ……………….. 106 pounds

- ATF-Acetone mix…………53 pounds

The ATF mentioned above is automatic transmission fluid (not a federal agency).

The ATF-Acetone mix is a “home brew” mix of 50 – 50 automatic transmission fluid and acetone. Note this “home brew” released bolts better than any commercial product in this one particular test. In some additional comments that I found while double-checking on the article I found that with some of the new ATF fluids it is hard to get a good mix. However if you add a small amount of diesel oil to the mixture this will help.